Mitigation measure RESources

The following are methods or techniques that may be applied to various operations or equipment when appropriate to mitigate estimated emissions.

Fugitive Dust Mitigation Measures

- SCAQMD, CEQA Handbook, Tables 11-4, page 11-15 and A11-9-A, page A11-77.

| Emission Source | Mitigation Measure | Emission Reduction Efficiency | Favorable Factors |
| --- | --- | --- | --- |
| Fugitive dust/ Construction | Apply non-toxic chemical soil stabilizers according to manufactures’ specifications, to all inactive construction areas (previously graded areas inactive for ten days or more) | 30%-65%* | Stabilizers applied in sufficient concentration to provide erosion protection for at least on year |
| Fugitive dust/ Construction | Replace ground cover in disturbed areas as quickly as possible | 15%-49%* | Small, densely planted ground cover |
| Fugitive dust/ Construction | Enclose, cover, water twice daily, or apply non-toxic soil binders, according to manufactures’ specifications, to exposed stockpiles (i.e., gravel, sand, dirt) with five percent or greater soil content | 30%-74%* | Automatic water mist or sprinkler systems should be installed in areas with stockpiles |
| Fugitive dust/ Construction | Water active sites at least twice daily | 34%-68%* | Water at sufficient frequency to keep soil moist enough so visible plumes are eliminated |
| Fugitive dust/ Construction | Suspend all excavating and grading operations when wind speeds (as instantaneous gusts) exceed 25 miles per hour | Not quantified |  |
| Fugitive dust/ Construction | Monitor for particulate emissions according to District-specified procedures | Not quantified |  |
| Fugitive dust from roads | All trucks hauling, dirt, sand, soil or other loose materials are to be covered, or should maintain at least two feet of freeboard in accordance with CVC Section 23114, (freeboard means vertical space between the top of the load and top of the trailer) | 7%-14%* | Tightly secured covering to truck |
| Fugitive dust from roads | Sweep streets once a day if visible soil materials are carried to adjacent streets (recommend water sweepers with reclaimed water ) | 25%-60%* | Sweep streets immediately after period of heaviest vehicular track-out activity |

| Emission Source | Mitigation Measure | Emission Reduction Efficiency | Favorable Factors |
| --- | --- | --- | --- |
| Fugitive dust from roads | Install wheel washers where vehicles enter and exit unpaved roads onto paved roads, or wash off trucks and any equipment leaving the site each trip. | 40%-70%* | Set up truck washing area on paved access road area so subsequent truck travel on unpaved roads can be eliminated |
| Fugitive dust from roads | Pave construction roads that have a truck volume of more than 50 daily trips by construction equipment, or 150 total daily trips for all vehicles | 92.5% (91% for trucks 94% for passenger vehicles) |  |
| Fugitive dust from roads | Pave construction access roads at least 100 feet onto the site from main road | 92.5% (91% for trucks 94% for passenger vehicles) |  |
| Fugitive dust from roads | Pave construction roads that have a daily traffic volume of less than 50 vehicular trips. | 92.5% (91% for trucks 94% for passenger vehicles) |  |
| Fugitive dust from roads | Apply water three times daily, or non-toxic stabilizers according to manufacturers’ specifications to all unpaved parking or staging areas or unpaved road surfaces | 45%-85%* | Use non-toxic chemicals that are formulated for unpaved road surfaces |
| Fugitive dust from roads | Traffic speeds on all unpaved roads to be reduced to 15 miles per hour or less | 40%-70%* | Effective traffic control |

*  Use the lowest value if better information is not known.  If higher than the lowest value is used, please provide the supporting analysis and data in the environmental documentation.

Combustion Emissions Mitigation Measures

Alternative Diesel Fuels

| Alternative Diesel Fuel | NOx | PM | HC | Toxics |
| --- | --- | --- | --- | --- |
| Aquazole fuela | 16% | 60% | 25% lower than diesel vehicle emission standard | No Increase |
| Clean Fuels Technology - water emulsified diesel fuelb | 15% | 58% | 25% lower than diesel vehicle emission standard | No Increase |
| O2diesel ethano-diesel fuel (O2Diesel)c | 1.6% | 20% | 25% lower than diesel vehicle emission standard | No Increase |
| a) Air Resources Board, Letter from Dean C. Simeroth to Dr. Phillippe Mulard, August 9, 2002. |  |  |  |  |
| b) Air Resources Board, Letter from Dean C. Simeroth to Dan Klaich, September 9, 2003. |  |  |  |  |
| c) Air Resources Board, Letter from Dean C. Simeroth to James Peeples, September 23, 2003. |  |  |  |  |